# SmartWatts Edge Gateway Installation Guide

# Complete Step-by-Step Guide for Beginners

Version: 1.0

Date: January 2025

Target Hardware: R501 RK3588 Edge AI Box (and other edge devices)

---

# Table of Contents

[Prerequisites](#prerequisites)

[Hardware Requirements](#hardware-requirements)

[Software Requirements](#software-requirements)

[Installation Methods](#installation-methods)

[Method 1: Direct Installation (Recommended)](#method-1-direct-installation-recommended)

[Method 2: Docker Installation](#method-2-docker-installation)

[Configuration](#configuration)

[Testing the Installation](#testing-the-installation)

[Connecting Devices](#connecting-devices)

10. [Troubleshooting](#troubleshooting)

11. [Maintenance](#maintenance)

12. [Support](#support)

---

# Prerequisites

## What You Need to Know

Basic understanding of Linux commands

Access to your edge device (R501 RK3588 or similar)

Internet connection for downloading software

Basic understanding of energy monitoring concepts

## What You Don't Need to Know

Advanced programming

Complex system administration

Deep networking knowledge

---

# Hardware Requirements

## Minimum Requirements

**CPU:** 4 cores (ARM64 or x86_64)

**RAM:** 4GB

**Storage:** 32GB microSD card or eMMC

**Network:** Ethernet or WiFi

**Power:** 5V/3A power supply

## Recommended Requirements

**CPU:** 8 cores (ARM64 or x86_64)

**RAM:** 8GB

**Storage:** 64GB+ microSD card or eMMC

**Network:** Gigabit Ethernet

**Power:** 5V/5A power supply

## Supported Devices

✅ R501 RK3588 Edge AI Box

✅ Raspberry Pi 4/5

✅ Orange Pi 5/5 Plus

✅ Jetson Nano/Orin

✅ Intel NUC

✅ BeagleBone

✅ Any Linux-based edge device

---

# Software Requirements

## Operating System

**Ubuntu Server 20.04 LTS** or newer

**Raspberry Pi OS** (for Raspberry Pi)

**Any Debian-based Linux distribution**

## Required Software

Python 3.11+

Git

Docker (optional)

MQTT Broker (Mosquitto)

SQLite3

---

# Installation Methods

You have two options for installation:

**Direct Installation** (Recommended for beginners)

**Docker Installation** (For advanced users)

---

# Method 1: Direct Installation (Recommended)

## Step 1: Prepare Your Edge Device

### 1.1 Connect to Your Device

# If using SSH (recommended)

ssh username@your-device-ip

# If using direct connection

# Connect keyboard, mouse, and monitor to your device

### 1.2 Update Your System

# Update package lists

sudo apt update

# Upgrade existing packages

sudo apt upgrade -y

# Install essential tools

sudo apt install -y curl wget git vim nano htop

### 1.3 Check Your Hardware

# Check CPU information

lscpu

# Check memory

free -h

# Check storage

df -h

# Check network

ip addr show

Expected Output:

CPU: 8 cores (for R501 RK3588)

Memory: 8GB available

Storage: At least 32GB free space

Network: Ethernet or WiFi connected

## Step 2: Download SmartWatts Edge Gateway

### 2.1 Create Installation Directory

# Create directory for SmartWatts

sudo mkdir -p /opt/smartwatts

sudo chown $USER:$USER /opt/smartwatts

cd /opt/smartwatts

### 2.2 Download the Software

# Method A: If you have the files on your computer

# Copy the edge-gateway folder to your device using SCP:

# scp -r edge-gateway/ username@your-device-ip:/opt/smartwatts/

# Method B: If downloading from repository

git clone https://github.com/your-repo/smartwatts-edge-gateway.git

cd smartwatts-edge-gateway

### 2.3 Verify Files

# Check that all files are present

ls -la

# You should see:

# - main.py

# - requirements.txt

# - config/

# - services/

# - deploy/

# - README.md

## Step 3: Run the Installation Script

### 3.1 Make Script Executable

# Navigate to the edge-gateway directory

cd edge-gateway

# Make installation script executable

sudo chmod +x deploy/install.sh

### 3.2 Run the Installation

# Run the installation script

sudo ./deploy/install.sh

What the script does:

Installs all required software packages

Creates a dedicated user account

Sets up the MQTT broker

Installs Python dependencies

Creates systemd service

Configures logging

Sets up backup system

### 3.3 Monitor Installation Progress

The installation will show progress messages like:

🚀 SmartWatts Edge Gateway Installation for R501 RK3588

============================================================

📦 Updating system packages...

📦 Installing system dependencies...

👤 Creating service user...

📁 Creating installation directory...

📥 Installing SmartWatts Edge Gateway...

🐍 Creating Python virtual environment...

📦 Installing Python dependencies...

📡 Configuring MQTT broker...

⚙️ Creating systemd service...

✅ SmartWatts Edge Gateway started successfully!

## Step 4: Verify Installation

### 4.1 Check Service Status

# Check if the service is running

sudo systemctl status smartwatts-edge

Expected Output:

● smartwatts-edge.service - SmartWatts Edge Gateway

Loaded: loaded (/etc/systemd/system/smartwatts-edge.service; enabled)

Active: active (running) since [timestamp]

Main PID: [process_id]

Tasks: [number] (limit: [limit])

Memory: [memory_usage]

CGroup: /system.slice/smartwatts-edge.service

### 4.2 Check API Health

# Test the API health endpoint

curl http://localhost:8080/api/v1/health

Expected Output:

{

"status": "healthy",

"timestamp": "2025-01-XX...",

"version": "1.0.0",

"hardware": "R501_RK3588",

"services": {

"mqtt": true,

"modbus": true,

"storage": true,

"device_discovery": true,

"ai_inference": true,

"data_sync": true

}

}

### 4.3 Check MQTT Broker

# Test MQTT broker

mosquitto_pub -h localhost -t test -m "Hello World"

mosquitto_sub -h localhost -t test

Expected Output:

Hello World

## Step 5: Access the Web Interface

### 5.1 Open Web Browser

Open your web browser and navigate to:

**API Health:** http://your-device-ip:8080/api/v1/health

**API Status:** http://your-device-ip:8080/api/v1/status

**Prometheus Metrics:** http://your-device-ip:9090/metrics

### 5.2 Test API Endpoints

# Get system status

curl http://localhost:8080/api/v1/status

# Get device list

curl http://localhost:8080/api/v1/devices

# Get MQTT statistics

curl http://localhost:8080/api/v1/mqtt/stats

---

# Method 2: Docker Installation

## Step 1: Install Docker

### 1.1 Install Docker

# Download and install Docker

curl -fsSL https://get.docker.com -o get-docker.sh

sudo sh get-docker.sh

# Add your user to docker group

sudo usermod -aG docker $USER

# Log out and log back in for changes to take effect

### 1.2 Install Docker Compose

# Install Docker Compose

sudo apt install docker-compose-plugin

### 1.3 Verify Docker Installation

# Check Docker version

docker --version

# Check Docker Compose version

docker compose version

## Step 2: Deploy with Docker

### 2.1 Navigate to Deploy Directory

cd /opt/smartwatts/edge-gateway/deploy

### 2.2 Start Services

# Start all services

docker compose up -d

# Check service status

docker compose ps

### 2.3 Monitor Logs

# View logs

docker compose logs -f

# View specific service logs

docker compose logs -f edge-gateway

---

# Configuration

## Step 1: Basic Configuration

### 1.1 Edit Configuration File

# Open configuration file

sudo nano /opt/smartwatts/config/edge-config.yml

### 1.2 Update Network Settings

network:

host: "0.0.0.0"  # Listen on all interfaces

port: 8080       # API port

cloud_api_url: "https://api.smartwatts.com"  # Your cloud API

cloud_api_key: "your-api-key-here"  # Your API key

### 1.3 Update MQTT Settings

mqtt:

broker_host: "localhost"

broker_port: 1883

username: null  # Set if authentication required

password: null  # Set if authentication required

## Step 2: Device Configuration

### 2.1 Add Modbus Devices

modbus:

enabled: true

devices:

name: "Solar Inverter"

type: "inverter"

address: 1

protocol: "tcp"

host: "192.168.1.101"  # Your inverter IP

port: 502

enabled: true

name: "Energy Meter"

type: "meter"

address: 2

protocol: "rtu"

port: "/dev/ttyUSB0"  # Serial port

baudrate: 9600

enabled: true

### 2.2 Add MQTT Devices

MQTT devices are automatically discovered. No configuration needed.

## Step 3: AI Model Configuration

### 3.1 Download AI Models

# Create models directory

sudo mkdir -p /opt/smartwatts/models

# Download models (if available)

# Place .tflite model files in this directory

### 3.2 Configure AI Settings

ai:

enabled: true

model_path: "/opt/smartwatts/models"

inference_interval_seconds: 60

batch_size: 32

confidence_threshold: 0.8

## Step 4: Restart Services

### 4.1 Restart Edge Gateway

# Restart the service

sudo systemctl restart smartwatts-edge

# Check status

sudo systemctl status smartwatts-edge

---

# Testing the Installation

## Step 1: Test API Endpoints

### 1.1 Health Check

curl http://localhost:8080/api/v1/health

### 1.2 System Status

curl http://localhost:8080/api/v1/status

### 1.3 Device Discovery

curl http://localhost:8080/api/v1/devices/discovered

## Step 2: Test MQTT Communication

### 2.1 Publish Test Message

# Publish a test message

curl -X POST http://localhost:8080/api/v1/mqtt/publish \

-H "Content-Type: application/json" \

-d '{

"topic": "test/energy",

"message": {"power": 1000, "voltage": 240}

}'

### 2.2 Subscribe to Messages

# Subscribe to test topic

mosquitto_sub -h localhost -t "test/energy"

## Step 3: Test Data Storage

### 3.1 Store Energy Reading

curl -X POST http://localhost:8080/api/v1/energy/readings \

-H "Content-Type: application/json" \

-d '{

"device_id": "test_device_01",

"device_type": "inverter",

"power": 2500.5,

"voltage": 240.2,

"current": 10.4

}'

### 3.2 Retrieve Energy Readings

curl "http://localhost:8080/api/v1/energy/readings?device_id=test_device_01"

---

# Connecting Devices

## Step 1: Connect Modbus Devices

### 1.1 Physical Connection

Connect RS485/USB adapter to your device

Connect Modbus devices to the adapter

Note the device IP addresses and serial ports

### 1.2 Configure in Edge Gateway

modbus:

devices:

name: "Your Inverter"

type: "inverter"

address: 1

protocol: "tcp"

host: "192.168.1.101"

port: 502

enabled: true

### 1.3 Test Connection

# Test Modbus device

curl http://localhost:8080/api/v1/modbus/read/Your_Inverter

## Step 2: Connect MQTT Devices

### 2.1 Configure Device

Configure your MQTT device to connect to:

**Broker:** Your edge device IP

**Port:** 1883

**Topics:** smartwatts/energy/{device_id}/data

### 2.2 Verify Connection

# Check discovered devices

curl http://localhost:8080/api/v1/devices/discovered

## Step 3: Connect Smart Plugs and Sensors

### 3.1 WiFi Configuration

Connect devices to your WiFi network

Ensure they can reach your edge device

### 3.2 MQTT Configuration

Configure devices to publish to MQTT broker

Use topic format: smartwatts/energy/{device_id}/data

---

# Troubleshooting

## Common Issues and Solutions

### Issue 1: Service Won't Start

# Check service status

sudo systemctl status smartwatts-edge

# Check logs

sudo journalctl -u smartwatts-edge -f

# Common solutions:

sudo systemctl restart smartwatts-edge

sudo systemctl daemon-reload

### Issue 2: API Not Responding

# Check if port is open

sudo netstat -tlnp | grep 8080

# Check firewall

sudo ufw status

# Allow port 8080

sudo ufw allow 8080

### Issue 3: MQTT Broker Issues

# Check MQTT broker status

sudo systemctl status mosquitto

# Check MQTT logs

sudo journalctl -u mosquitto -f

# Restart MQTT broker

sudo systemctl restart mosquitto

### Issue 4: Database Issues

# Check database file

ls -la /opt/smartwatts/data/

# Check database integrity

sqlite3 /opt/smartwatts/data/edge.db ".schema"

# Repair database if needed

sqlite3 /opt/smartwatts/data/edge.db ".recover" | sqlite3 /opt/smartwatts/data/edge_recovered.db

### Issue 5: Permission Issues

# Fix ownership

sudo chown -R smartwatts:smartwatts /opt/smartwatts

# Fix permissions

sudo chmod -R 755 /opt/smartwatts

## Getting Help

### Check Logs

# Edge Gateway logs

sudo journalctl -u smartwatts-edge -f

# MQTT broker logs

sudo journalctl -u mosquitto -f

# System logs

sudo journalctl -f

### Monitor System Resources

# CPU and memory usage

htop

# Disk usage

df -h

# Network connections

netstat -tlnp

---

# Maintenance

## Daily Tasks

### Check Service Status

# Quick status check

sudo systemctl status smartwatts-edge

# Detailed monitoring

/opt/smartwatts/monitor.sh

### Check Logs

# Recent errors

sudo journalctl -u smartwatts-edge --since "1 hour ago" | grep ERROR

# Recent warnings

sudo journalctl -u smartwatts-edge --since "1 hour ago" | grep WARNING

## Weekly Tasks

### Backup Data

# Manual backup

/opt/smartwatts/backup.sh

# Check backup status

ls -la /opt/smartwatts/backups/

### Update System

# Update system packages

sudo apt update && sudo apt upgrade -y

# Restart services

sudo systemctl restart smartwatts-edge

## Monthly Tasks

### Clean Old Data

# Clean data older than 30 days

curl -X POST http://localhost:8080/api/v1/maintenance/cleanup?days=30

### Check Disk Space

# Check disk usage

df -h

# Clean up if needed

sudo apt autoremove -y

sudo apt autoclean

## Update SmartWatts Edge Gateway

### Update Code

# Navigate to installation directory

cd /opt/smartwatts

# Stop service

sudo systemctl stop smartwatts-edge

# Backup current installation

./backup.sh

# Update code (if using git)

git pull origin main

# Update dependencies

source venv/bin/activate

pip install -r requirements.txt

# Start service

sudo systemctl start smartwatts-edge

---

# Support

## Documentation

**README:** /opt/smartwatts/README.md

**API Documentation:** http://your-device-ip:8080/docs

**Configuration:** /opt/smartwatts/config/edge-config.yml

## Monitoring Tools

**System Monitor:** /opt/smartwatts/monitor.sh

**Prometheus Metrics:** http://your-device-ip:9090/metrics

**Grafana Dashboard:** http://your-device-ip:3000

## Getting Help

Check the troubleshooting section above

Review the logs for error messages

Check the system resources

Contact support with specific error messages

## Useful Commands

# Service management

sudo systemctl start smartwatts-edge

sudo systemctl stop smartwatts-edge

sudo systemctl restart smartwatts-edge

sudo systemctl status smartwatts-edge

# Log viewing

sudo journalctl -u smartwatts-edge -f

sudo journalctl -u mosquitto -f

# Monitoring

/opt/smartwatts/monitor.sh

htop

df -h

free -h

# API testing

curl http://localhost:8080/api/v1/health

curl http://localhost:8080/api/v1/status

---

# Conclusion

Congratulations! You have successfully installed and configured the SmartWatts Edge Gateway. Your edge device is now ready to:

✅ Monitor energy consumption

✅ Connect to smart devices

✅ Run AI inference

✅ Sync data with the cloud

✅ Provide real-time insights

The system is designed to be robust and self-maintaining, but regular monitoring and maintenance will ensure optimal performance.

For additional support or questions, please refer to the troubleshooting section or contact the support team.

---

End of Installation Guide